Cat 6A U/UTP Patch Cable, 26 AWG,

Pure Bare Copper

Model: xxx

Main Features:

- Stranded Pure Bare Copper, 26 AWG;

- Supports CAT6A U/UTP bandwidth up to 500 MHz;

- 1U gold-plated contacts for optimized signal integrity;

- 4 Twisted Pairs with filler reduce crosstalk, noise, and interference;

- HDPE Insulation enhances signal integrity and reduces signal loss;

- Fluke Tested for Reliable Performance.

| Product Type |  |
| --- | --- |
| Category | Cat 6A |
| Connector | 2x RJ45 Male (8P8C, 1U Gold Plating) |
| Shielding | Unshielded (U/UTP) |
| Conductor |  |
| Conductor Type | Stranded |
| Conductor Material | Pure Bare Copper |
| Conductor Size | 7/0.16±0.008 mm |
| Wire Gauge | 26 AWG |
| Conductor Quantity | 4 Twisted Pairs |
| Wire Code | 1. Blue / White-Blue 2. Orange / White-Orange 3. Green / White-Green 4. Brown / White-Brown |
| Insulation |  |
| Insulation Material | HDPE |
| Minimum Average Thickness | 0.12 mm |
| Outer Diameter | 0.89±0.08 mm |
| Filler |  |
| Material | HDPE |
| Jacket |  |
| Material | PVC |
| Minimum Average Thickness | 0.51 mm |
| Outer Diameter | 6.0±0.2 mm |
| Electrical Properties |  |
| Standard Bandwidth | 500 MHz |
| Input Impedance | 100±15% Ω (@4~100 MHz); 100±22% Ω (@100~250 MHz) |
| Propagation Delay | ≤ 548 ns/100m |
| Propagation Skew | ≤ 45 ns/100m |
| Conductor Resistance | ≤ 149 Ω/km @20°C |
| Conductor Resistance Unbalance | ≤ 5% |
| Insulation Resistance | ≥ 5000 MΩ·km |
| Mechanical Properties |  |
| Min. Bending Radius | 8× Cable OD |
| Environment |  |
| Operating Temperature | -15°C to 70°C |
| Operating Humidity | RH 10% to 90% (non-condensing) |
| Storage Temperature | -40°C to 80°C |
| Storage Humidity | RH 5% to 96% (non-condensing) |
| General |  |
| Color | Black |
| Length | 0.25/0.5/1/2/5/10/20 meter |
| Diagram | (unit: mm) |